- Course help - David Moore - 01626 331351 - dcmoore@timber.org.uk

YOUR ANSWER TO ROT
Exercise 2

  You are standing inside a roof with the following tools: a torch, a tape measure, a 'damp' testing meter, a screwdriver, plus a pad and pencil.

(Use the attached form to make notes as you go and then compare them with the suggested answers).

- What you can see

Sloping timbers (common rafters) coming down from the top (ridge) to sit at the bottom on the wall. They are sitting on another piece of wood which is a horizontal beam across the building (tie beam or collar).

- Checking for defects

You notice that the Tie Beam looks creased and darkened at the end where it sits on the wall.

Q.  What other symptoms of decay or attack might you notice?

A.

Q. Would these differ at all if the roof was made of a softwood (Pine) or a hardwood (say, Oak)?

A.

- Look for the cause

Q. What is causing the creasing and darkening?

A.

Q. How are you going to confirm that the cause is still present?

A.

Q. Suggest the 'moisture meter' readings on which you might advise a Client as follows: a)no action required, b)at risk, needs treatment

A. a) below ……..%  b) above ………%

Q. Give as many examples as possible of where could the cause be coming from?

A.

- Assess the damage

Q. How would you decide to what depth the timber is damaged?

A.

Q. Approximately what proportion of the timber can be damaged and surface repaired, without needing to cut out the section?

A.

Q. Is damage more important in one part of the timber than another, from a structural point of view?

A.

Always recommend a Structural Engineer if you see any signs of structural alteration or movement.

- Offer a repair

In one area there is slight surface damage: when dug out the missing wood measures 1 inch (25mm) x 2 inches (50mm) x 4 inches (100mm).

Q.  What volume in cc's or litres of repair product will you need and what type of product could you use?

A.

The bottom 3 foot 3 inches (1 metre) of the 12 inch (300mm) x 10 inch (250mm) Tie Beam is beyond repair.

Q. Which timber repair technique could you use to replace it?

A.

Q. Which types of resin products are you likely to use?

A.

Q. What techniques could be used to cut slots or holes into the remaining timber, for connection to a new piece?

A.

Q. How much do you think the repair materials would cost?

A. For a 12 x 10 inch timber x 1 metre cut off made in softwood about £………

Q. How long do you think it would take one man to cut out and replace one Tie Beam end, assuming the Rafter foot was sound?

A. Excluding travel, access, propping and making good about ………. hours.

Q. What types of treatment could you offer, to minimise the risk of future rot?

A.

Q. What other measures might you take, depending on the type of roof coverings, to reduce the risk of mould growth and insect attack?

A.

Course  help - David Moore - 01626 331351 - dcmoore@timber.org.uk

Site: ©1998 D C Moore
Page Design:- dcmoore@timber.org.uk