- Course help - David Moore - 01626 331351 - dcmoore@timber.org.uk

YOUR ANSWER TO ROT
Exercise 3

  You are standing on the first or ground floor of a building with the following tools: a torch, a tape measure, a 'damp' testing meter, a screwdriver, plus a pad and pencil.

(Use the attached form to make notes as you go and then compare them with the suggested answers).

- What you can see

Floor boards laid over timber joists. The joists may be supported in the middle by a larger 'carrier' beam.

- Checking for defects

You notice that the joists look creased and light coloured  where they disappear into the wall (wall socket or bearing).

Q.  What other symptoms of decay or attack might you notice?

A.

Q. Would these differ at all if the floor was made of a softwood (Pine) or a hardwood (say, Oak)?

A.

- Look for the cause

Q. What is causing the creasing and lightening of the bearing ends?

A.

Q. How are you going to confirm that the cause is still present?

A.

Q. Suggest the 'moisture meter' readings on which you might advise a Client as follows: a)no action required, b)at risk, needs treatment

A. a) below …….%  b) above …….%

Q. Give as many examples as possible of where could the cause be coming from?

A.

- Assess the damage

Q. How would you decide to what depth the timber is damaged?

A.

Q. Approximately what proportion of the timber can be damaged and surface repaired, without needing to cut out the section?

A.

Q. Is damage more important in one part of the Joist than another, from a structural point of view?

A.

Always recommend a Structural Engineer if you see any signs of structural alteration or movement.

- Offer a repair

In one area there is slight surface damage: when dug out the missing wood measures 1 inch (25mm) x 1 inches (25mm) x 4 inches (100mm).

Q.  What volume in cc's or litres of repair product will you need and what type of product could you use?

A. ……..cc or ……….litres  of Mouldable Epoxy Mortar.

The bottom 1 foot (300mm) of 4 of the 8 inch (200mm) x 3 inch (75mm) Joist Ends are beyond repair.

Q. Which timber repair technique could you use to replace them?

A.

Q. Which types of resin products are you likely to use?

A.

Q. What techniques could be used to cut slots or holes into the remaining timber, for connection to a new piece?

A.

Q. How much do you think the repair materials would cost?

A. For an 8 x 3 inch timber x 300mm 45 degree cut off made in softwood about £…….

Q. How long do you think it would take one man to cut out and replace one rafter foot?

A. Excluding travel, access, propping and making good about ……. hours.

Q. What types of treatment could you offer, to minimise the risk of future rot?

A.

Q. What other measures might you take, depending on the type of floor and ceiling materials, to reduce the risk of rot and insect attack?

A.

Course  help - David Moore - 01626 331351 - dcmoore@timber.org.uk

Site: ©1998 D C Moore
Page Design:- dcmoore@timber.org.uk